DevCtrl_API Manual

- Download DevCtrl_Api.zip and unzip to local folder ( as D:\DevCtrl_Api ).

http://version.zswxyd.com/devctrl_api/DevCtrl_API.zip

< open the website, download the DevCtrl_API.zip file >

< Open the folder of download path >

< Find the download file at download folder >

< unzip the Devctrl_API.zip to local folder,  C:\WXYD >

- Run DevCtrl_Api.exe and login.

- Please obtain your username and password from us, using email  <james@zswxyd.com> or mobile (+86 13928127700).

- Please write down the IP Address of the computer，example 172.16.0.5

- *** You can connect to a local computer instead of a cloud computer, and we will explain this in detail in our subsequent steps

- < run the execute file “DevCtrl_API_Debug.exe” >

- < Login with User Name and password. No user? Please call me >

- < run cmd.exe open command window >

- < run cmd.exe open command window, and run ipconfig to get the your PC IP Address >

- Turn off the Firewall of Server whick running “DevCtrl_Api.exe”. Port=1883

< the card reader need TCP-Port 1883 connect to DevCtrl_Api Program  >

- Setting your WIFI router. Need two SSID:

  - WIFI SSID Setting

- SSID: DVESPUB,  Password: DS123456

- SSID: DVESWIFI,  Password: 6220asWF

- *** You can modify your SSID and password, and we will explain this in detail in our subsequent steps

  - Network

- Router LAN IP Address: 172.16.0.1

- DHCP: 172.16.0.50 - 172.16.8.255

- Mask: 255.255.0.0

  - Sever IP Address: 172.16.0.5

- Start to power on the card reader.

- *** The default settings for the card reader are

- “SSID=DVESWIFI, Password=6220asWF, Server=172.16.0.5”

- API Test

  - Prepare your http server, not https server.

  - Tell Devctrl_Api program your http server, port (if have), and function, and data format ( Recommend using JSON format).

- < Double click to make changes, apply to save >

- Data Format:

- Raw:  card_no=1234567890&mach_sn=66621234&tickets=0&tokens=0&xid=4256&event=load_card

- Json:  {“card_no”:”1234567890”,”mach_sn”:”66621234”,”xid”:4256,”event”:”load_card”}

  - Test your API by hand. You can just test your API with no card-reader.

- ( setup the reader_sn and card_no, click the function button )

  - Test API API by card reader.

- API description

- Require Json:

- {

- “card_no”:”1234567890”,	// card no

- ”mach_sn”:”66621234”,		// card reader sn

- ”tickets”:0,				// tickets to add, negative for sub

- ”tokens”:0,				// tokens to add or sub, negative for sub

- ”xid”:4256,				// trans-id, check

- ”event”:”load_card”,		// sub function

- “app_id”:””,  				// *** app_id

- “time”:””,  				// *** timestamp

- “sign”:””  				// *** md5_sign

- }

- Reply Json:

- {

- “card_no”:”1234567890”,	// card no

- ”mach_sn”:”66621234”,		// card reader sn

- "eToken":5443,			// card balance of coin

- "eTicket":456, 			// card balance of ticket

- "message":"Happy Birthday",	// message show on reader screen

- "status":"0"				// result:  =0 success, <>0 fail

- “time”:””,  				// *** timestamp

- “sign”:””  				// *** md5

- }

- ***Status: 	=0  success,   <>0 fail

- ***message: show on card reader screen, tell customer why fail or success welcome.

- ***app_id, timestamp, sign     no need

| - API | - Require Json | - Reply Json | - Remark |
| --- | --- | --- | --- |
| - for_test | - {”event”:”for_test”} | - Same as require | - Just a test |
| - dev_online | - { - ”mach_sn”:”66621234”, - ”event”:”dev_online” - } | - { - ”mach_name”:”name”, - ”type”:”0”, - “price1”:XX.X, - “price2”:YY.Y, - “signal1”:XX, - “signal2”:YY, - "mach_name":"mach_name", - "message":"message on reader", - "status":"0" //success - } | - Heartbeat - Type - =0 Game - =1 Kiosk - =2 Gate - =4 Sale - =5 Time |
| - load_card | - { - “card_no”:”1234567890”, - ”mach_sn”:”66621234”, - ”event”:”load_card” - } | - { - “card_no”:”1234567890”, - "eToken":1000, // balance1 - "eTicket":2000, // balance2 - "message":"message on reader", - "status":"0" //success - } | - Member balance |
| - use_coin | - { - “card_no”:”1234567890”, - ”mach_sn”:”66621234”, - ”tokens”:-100, - ”event”:”use_coin” - } | - { - “card_no”:”1234567890”, - "eToken":900, // new balance1 - "eTicket":2000, // balance 2 - "message":"message on reader", - "status":"0" - } | - Sub coin - from card - by game |
| - out_ticket | - { - “card_no”:”1234567890”, - ”mach_sn”:”66621234”, - ”tickets”:200, - ”event”:”out_ticket” - } | - { - “card_no”:”1234567890”, - "eToken":900, // balance1 - "eTicket":2200, // new balance2 - "message":"message on reader", - "status":"0" - } | - Add ticket - to card - by game |
| - out_coin | - { - “card_no”:”1234567890”, - ”mach_sn”:”66621234”, - ”tokens”:10, - ”event”:”out_coin” - } | - { - “card_no”:”1234567890”, - "eToken":910, // new balance1 - "eTicket":2200, // balance2 - "message":"message on reader", - "status":"0" - } | - Add coin - to card - by game |
| - save_ticket | - { - “card_no”:”1234567890”, - ”mach_sn”:”66621234”, - ”tickets”:500, - ”event”:”save_ticket” - } | - { - “card_no”:”1234567890”, - "eToken":910, // balance1 - "eTicket":2700, // new balance2 - "message":"message on reader", - "status":"0" - } | - Add ticket - to card - by kiosk |
| - save_coin | - { - “card_no”:”1234567890”, - ”mach_sn”:”66621234”, - ”tokens”:50, - ”event”:”save_coin” - } | - { - “card_no”:”1234567890”, - "eToken":960, // new balance1 - "eTicket":2700, // balance2 - "message":"message on reader", - "status":"0" - } | - Add coin - to card - by kiosk |
| - real_in_coin | - { - ”mach_sn”:”66621234”, - ”tokens”:50, - ”event”:”real_in_coin” - } | - { - ”mach_sn”:”66621234”, - "message":"message on reader", - "status":"0" - } | - In real coin - from game |
| - real_out_ticket | - { - ”mach_sn”:”66621234”, - ”tickets”:200, - ”event”:”real_out_ticket” - } | - { - ”mach_sn”:”66621234”, - "message":"message on reader", - "status":"0" - } | - Out real ticket - by game |
| - real_out_coin | - { - ”mach_sn”:”66621234”, - ”tokens”:200, - ”event”:”real_out_coin” - } | - { - ”mach_sn”:”66621234”, - "message":"message on reader", - "status":"0" - } | - Out real coin by game |
| - real_out_gift | - { - ”mach_sn”:”66621234”, - ”gifts”:1, - ”event”:”real_out_gift” - } | - { - ”mach_sn”:”66621234”, - "message":"message on reader", - "status":"0" - } | - Out gift - by game |
| - check_password | - { - “card_no”:”1234567890”, - ”mach_sn”:”66621234”, - ”password”:”123456”, - ”event”:”check_password” - } | - { - “card_no”:”1234567890”, - "eToken":910, // balance1 - "eTicket":2200, // balance2 - "message":"message on reader", - "status":"0" - } | - Check - card password - by Kiosk |
| - exchange_coin | - { - “card_no”:”1234567890”, - ”mach_sn”:”66621234”, - ”tokens”:-200, - ”event”:”exchange_coin” - } | - { - “card_no”:”1234567890”, - "eToken":710, // new balance1 - "eTicket":2200, // balance 2 - "message":"message on reader", - "status":"0" - } | - Dispense coin - from card - by kiosk |
| - exchange_coin_rpt | - { - “card_no”:”1234567890”, - ”mach_sn”:”66621234”, - ”tokens”:0, - ”event”:”exchange_coin_rpt” - } - tokens=0 dispense finish - tokens>0 Insufficient, refund to card | - { - “card_no”:”1234567890”, - "eToken":710, // new balance1 - "eTicket":2200, // balance 2 - "message":"message on reader", - "status":"0" - } | - Result of dispense coin - by kiosk |

- Check your card reader status

- Starting,

- < Lang, Direction, Network, Logo, Firmware version, SN >

- Connecting to Network ( Work-SSID, Public-SSID)

- Try to connecting Work-SSID

- Success to connected to Work-SSID, skip to step-3.

- No Work-SSID network, find new network config < new Work-SSID>  from Public-SSID.

- Reboot, Connecting to new Work-SSID.

- Connecting to Server

- Standby

- Load Card

- Use coin from card

- Plug-in type

- Swipe card type

- Out ticket from Game

- Cache out ticket: First out the ticket to the card reader, then choose to save to card or out the real ticket from card reader.

- Only out real-ticket. All game ticket out by real ticket, no e-ticket.

- Only out e-ticket. All game ticket out to last card, no real ticket.

- Auto out real-ticket or e-ticket. Insert card then out e-ticket, no card inserted out real ticket.

- Choose how to handle the cache ticket in card reader. Save to card, or just out a real ticket.

- Change the card reader setting with Function-Card